SOP for FS-Avoidance behavior chambers built by Zengyou

- Environmental Habituation

  - Bring the mice to the testing room and allow them to habituate to the environment for at least 30 minutes.

- Turn On Equipment

  - Turn on the computer and cameras using the power switch.

- Set Up Camera Monitoring

  - Open Blue Iris and confirm the camera views for Boxes 5–8.

- Set Up Med-PC

  - Open Med-PC.

  - Enter/check the initial parameters for each cage, including the subject ID and protocol.

- Pre-Test Check
Before starting, confirm:

  - Syringe pump: Ensure 0.2% saccharin solution is loaded and all tubing is filled.

  - FS device: Confirm the setting is approximately 0.3 mA.

- Start Behavioral Testing

  - Place the mice in the behavioral chambers.

  - Start the appropriate Med-PC program for each mouse.

- After Testing

  - Return the mice to their home cages.

  - Record the number of rewards and FS count for each mouse.

  - Measure and record the remaining volume of saccharin solution.

- Clean the Chambers

  - Clean the chambers with 70% ethanol.

  - Allow the chambers to dry before starting the next batch.